# Cubic-Equivariant k-Space Convolution and a Differentiable Bond-Valence Field for Ionic-Conductivity Prediction in Solid-State Electrolytes

## Highlights

- k-SEC: cubic-equivariant reciprocal-space encoder for crystal property regression.

- DBVF: end-to-end-learnable bond-valence module embedded in a neural network.

- OBELiX neural-method MAE: 1.047 standalone, 0.980 stacked (n = 281, 5-fold CV).

- DBVF features alone do not help LightGBM — the gain is architectural, not tabular.

- Unsupervised top-15 screen of 18,574 MP crystals recovers known fast-Li families.

## Graphical abstract

A two-stream architecture in which a reciprocal-space encoder built on cubic-harmonic directional filters and a differentiable bond-valence field operating on real-space Li environments are trained jointly, end-to-end, to predict log₁₀σ of solid-state Li-ion electrolytes. A virtual screen on 18,574 Materials Project crystals identifies the four canonical fast-Li-ion conductor families in its top-15 without family-level supervision.

## Abstract

Predicting the room-temperature ionic conductivity σ of solid-state lithium-ion electrolytes from crystal structure remains a bottleneck for materials discovery. I introduce two architectural primitives. k-SEC is a neural encoder whose feature maps live in reciprocal space throughout the network and whose cubic-group equivariance is enforced by construction via cubic-harmonic directional filters and a cross-shell gated attention modulated only by k-magnitudes. DBVF (Differentiable Bond-Valence Field) embeds Brown’s bond- valence sum into the network as a learnable module — the first treatment of Brown’s tabulated parameters as trainable parameters of a neural network. On the OBELiX benchmark (281 labelled solid electrolytes, 5-fold cross-validation × 5 seeds) the joint model attains MAE 1.047 [95 % CI 0.925–1.183] standalone and 0.980 in a stacked ensemble with a gradient-boosted-tree baseline; Spearman ρ = 0.78 with 70 % top-10 precision. An unsupervised screen of 18,574 Li-containing Materials Project crystals identifies all four known fast-Li-ion conductor families in its top-15. A control experiment shows the DBVF gain is realised only through end-to-end training; a learning-curve experiment shows a 45 % gap reduction to the tabular baseline as training samples grow from 90 to 135 per fold, before plateauing at OBELiX scale.

## 1. Introduction

Solid-state electrolytes (SSEs) are the central enabling component of high-energy-density solid-state Li-ion batteries. The room-temperature ionic conductivity σ at 298 K is the figure-of-merit that distinguishes device-relevant materials from the rest of the design space. Direct DFT treatment of σ — through nudged-elastic-band calculations of Li migration barriers or molecular dynamics of Li diffusion — is computationally prohibitive at the screening scale of 10⁴–10⁵ candidate compositions. A predictive σ-from-structure model would therefore be transformative.

The published landscape divides into three regimes. Tabular models operate on elemental and structural descriptors (Magpie, Matminer, Roost) and dominate at small n on most materials-property benchmarks [1-3]. Real-space neural models — CGCNN [4], ALIGNN [5], MEGNet [6], M3GNet [7] — encode crystals as atom-centred message-passing graphs and inherit periodicity only via distance cutoffs and data augmentation. Equivariant potentials — NequIP [8], MACE [9], CHGNet [10] — pretrain on DFT energies, forces and stresses and provide foundation features through single-point evaluations.

For a property as sensitive to the periodic Li sub-lattice and to local anion coordination as σ, two limitations of the current literature are salient. First, no published architecture builds a primary representation in reciprocal space while enforcing space-group equivariance by construction; ReGNet [11] is the closest relative but uses a single FFT–filter–IFFT block as an auxiliary decoration on a real-space GNN, with no Brillouin-zone symmetry enforcement. Second, the most physically interpretable handle on Li mobility — Brown’s bond-valence sum [12] — has been used only as a post-hoc filter or as a tabulated descriptor, never as a learnable sub-module that participates in end-to-end gradient flow.

I introduce two architectural primitives to address both gaps.

(i) k-SEC (k-space equivariant convolution). The encoder builds atomic structure factors F(k) on a cubic-symmetrised wavevector grid; all feature maps live in ℂ^(K × D) throughout the network. Two novel components ensure that cubic-group equivariance is enforced by construction rather than learned approximately: (a) cubic-harmonic directional filters W(|k|, K₀, K₄ₐ, K₄ᵦ, K₆ₐ, K₆ᵦ) whose argument is a 6-d encoding of k that is invariant under O_h rotations, and (b) cross-shell gated attention, a full K×K attention modulated by a gate that depends only on the magnitudes (|k_i|, |k_j|, Δ|k|), hence preserving cubic-equivariance while permitting umklapp-style cross-shell coupling.

(ii) DBVF (Differentiable Bond-Valence Field). For each Li site I compute the bond-valence sum V(r_Li) = Σ_anion exp((r₀ − d)/b) with (r₀, b) instantiated as learnable per-anion parameters, and pool the resulting per-site distribution into an 8-d crystal-level descriptor that is consumed by the readout. To my knowledge no prior model has end-to-end-learned (r₀, b) inside a neural network; the closest work [13,14] uses Brown-table values as fixed inputs to KMC pipelines.

On the OBELiX benchmark — a curated set of solid-state Li-ion electrolytes [15] — the joint k-SEC + DBVF model attains MAE 1.047 standalone and 0.980 in a stacked ensemble with a tabular-feature LightGBM. To my knowledge this is the best MAE reported by a neural model on the OBELiX 281-sample filter, although direct comparison to published numbers is complicated by varying sample filters; for an apples-to-apples comparison I re-evaluate CGCNN-lite and a prior dual-graph baseline on the same filter and report a 25–33 % MAE reduction (Section 4.1). I also include a controlled-comparison test that disentangles architectural novelty from feature novelty: extracting DBVF features from a trained model and feeding them to LightGBM does not improve the tree (and slightly worsens it), supporting the interpretation that DBVF contributes through end-to-end training-time gradient flow rather than as a static descriptor.

Three honest limitations frame the contribution. (1) The OBELiX benchmark is small (n = 281 after physical-σ filtering), and a gradient-boosted tree on hand-crafted Magpie + lattice + geometric features attains MAE 0.924 — a strong baseline that the standalone neural model does not surpass. (2) Increasing the k-grid resolution beyond n_max = 2 exhausts consumer-GPU memory because cross-shell attention is O(K²); this is an architectural limit at the current implementation stage. (3) Transfer of the Li-pretrained k-SEC encoder to the elasticity-target Matbench task log_gvrh underperforms published real-space encoders, indicating that the encoder’s pretraining is domain-specialised rather than universally transferable.

## 2. Related work

### 2.1 Real-space crystal-property neural networks

CGCNN [4] established atom-graph convolution; ALIGNN [5] added angle terms; MEGNet [6] introduced global state inputs; M3GNet [7] folded three-body terms into a unified potential. All operate in real space and treat periodicity through distance-cutoff graphs.

### 2.2 Reciprocal-space neural representations

Fourier neural operators [16] popularised spectral architectures for PDE surrogates. ReGNet [11] is the closest published relative to k-SEC: it uses a single FFT block as auxiliary input alongside a real-space GNN. My work differs in three respects: (i) the primary representation lives in k-space throughout the network; (ii) cubic-group equivariance is enforced through the cubic-harmonic basis, not learned implicitly; (iii) cross-shell gated attention permits umklapp-style cross-shell coupling without breaking equivariance.

### 2.3 Equivariant potentials and foundation features

NequIP [8], MACE [9], MACE-MP-0 [17] and CHGNet [10] provide SE(3)-equivariant potentials whose single-point evaluations have been used as input features for downstream property prediction. I compare against MACE-MP-0 features as a strong foundation-model baseline (Section 4.5) and find that DBVF — a σ-targeted, end-to-end-learnable physics module — slightly outperforms MACE features at this benchmark.

### 2.4 Ionic-conductivity-specific machine learning

Hargreaves et al. [18] released the Liverpool σ dataset (641 entries, no CIFs); Pizarro et al. [15] released OBELiX (562 entries, 285 with CIFs). Prior σ-prediction work has used CrabNet- style composition-only transformers [18,19] and fragment-KMC + MLIP pipelines [13]. None deploy a reciprocal-space primary encoder or an end-to-end-learnable bond-valence module.

### 2.5 Bond-valence theory in materials informatics

Brown’s bond-valence sum [12] is a 70-year-old crystallographic tool used post-hoc to identify low-energy Li sites and migration paths [14,20]. The (r₀, b) parameters are tabulated. DBVF is, to my knowledge, the first treatment of (r₀, b) as learnable, gradient-flowing parameters of a neural module.

## 3. Methods

### 3.1 Notation

A crystal C = (L, {(Z_j, r_j)}) consists of a lattice matrix L ∈ ℝ³ˣ³ and atoms with atomic numbers Z_j and fractional coordinates r_j ∈ [0,1)³. The target is y = log₁₀ σ at 298 K (units S cm⁻¹).

### 3.2 k-SEC encoder

Wavevector grid. I construct a cubic-symmetrised grid {k_m} ⊂ ℤ³ of integer Miller indices with |m_i| ≤ n_max, and group points into shells of equal |k|. I use n_max = 2 (K ≈ 33 unique k-points) throughout; n_max = 3 (K ≈ 80) exceeds 12-GB GPU memory because cross-shell attention is O(K²).

Atomic structure factors. Given learned per-channel atomic embeddings z_j ∈ ℂ^D,

F_c(k_m) = Σ_j z_{j,c} · exp(−2πi k_m · r_j) ∈ ℂ^{K × D}

provides the input feature map. Periodicity is built in by construction because F is the discrete Fourier transform of the atomic density.

Cubic-harmonic directional filters. A purely radial filter W(|k|) discards direction. To retain direction while preserving cubic equivariance I evaluate five O_h-invariant polynomial invariants on the unit sphere:

K_0 = 1 (l = 0)
K_{4a} = x⁴ + y⁴ + z⁴ − 3/5 (l = 4, primary cubic invariant)
K_{4b} = x²y² + y²z² + z²x² − 1/5 (l = 4, mate)
K_{6a} = x⁶ + y⁶ + z⁶ − 3/7 (l = 6)
K_{6b} = x²y²z² − 1/105 (l = 6, triple)

These have zero mean over the sphere and span the lowest cubic-invariant irreducible representations beyond the trivial scalar. The filter is parameterised by a 6→D MLP

W(|k|, K_0(k̂), K_{4a}(k̂), K_{4b}(k̂), K_{6a}(k̂), K_{6b}(k̂))

and is cubic-equivariant by construction because every invariant commutes with every O_h rotation of k̂.

Cross-shell gated attention. Shell-restricted attention preserves cubic equivariance trivially (each shell is closed under O_h) but blocks physical inter-shell coupling. I instead use full K×K attention with each edge modulated by a learned gate g(|k_i|, |k_j|, Δ|k|) ∈ [0,1]. Crucially, the gate depends only on the magnitudes of the wavevectors, which are cubic-scalar invariants, so applying any cubic rotation R ∈ O_h to the input crystal leaves the gate values unchanged. The values, keys, and queries are derived from the cubic-equivariant features of Section 3.3 and therefore transform covariantly with R; combining a covariant attention pattern with an invariant gate produces a covariant output. Formally, if F̃ denotes the feature map after applying R to the input crystal, then g(R) · attention(F̃) = R · [g · attention(F)] by linearity of the attention sum and the invariance of g, so the layer is O_h-equivariant by construction. The gate is learned per-layer and per-head and converges, in my trained models, to a smooth function of Δ|k| that decays with shell distance — i.e. it learns when to behave as shell-restricted (small Δ|k|) versus umklapp-like (large Δ|k|), without breaking equivariance.

### 3.3 Differentiable Bond-Valence Field (DBVF)

For each Li site r_Li in the unit cell, Brown’s bond-valence sum is

V(r_Li) = Σ_{anion ∈ N(r_Li, R_cut)} exp((r₀_anion − d_{Li,anion}) / b_anion)

where (r₀_anion, b_anion) are the bond-valence parameters and the sum runs over anions within a real-space cutoff R_cut = 4.0 Å.

In DBVF, (r₀, b) are learnable per-anion parameters of a neural module and are updated by backpropagation. I initialise from Brown’s 2002 table where available (O, S, Se, F, Cl, Br, I, N) and apply softplus reparameterisation r₀ = softplus(r₀_raw), b = softplus(b_raw) to enforce positivity. Per-Li valences {V(r_Li)} are pooled into an 8-d crystal-level descriptor (mean, std, min, max, 25/50/75 percentiles, n_Li) that feeds the readout MLP. The full computation is differentiable end-to-end so gradients flow through (r₀, b).

To my knowledge, no published crystal-property prediction architecture treats Brown’s bond-valence parameters as learnable model parameters; the closest related work uses tabulated (r₀, b) as fixed inputs to KMC pipelines [14,20] or as bond-valence-energy landscape descriptors [13]. I position DBVF as the first architectural treatment of bond valence — Brown’s parameters become model parameters that the network can adjust to the σ objective while preserving the underlying physical functional form.

### 3.4 Joint readout

The k-SEC encoder produces a (B, K, D) reciprocal-space feature map; mean pooling across K yields a (B, D) crystal embedding. The DBVF module produces a (B, 8) valence descriptor. The two are concatenated with hand-crafted descriptors (Magpie 132-d, lattice 6-d, Wyckoff geometric 20-d) and consumed by a 3-layer MLP regressor with dropout 0.15 trained to log₁₀σ.

### 3.5 Pretraining and fine-tuning

The k-SEC encoder is pretrained on 18,574 Li-containing Materials Project crystals to predict formation energy per atom (val MAE 0.072 eV/atom, 6 epochs, batch 32). Pretrained weights are loaded into the OBELiX fine-tuning model; the DBVF module is initialised fresh and trained jointly with the encoder.

I considered, and report on, an alternative broad-domain pretraining corpus of 218,057 crystals from Materials Project (full), JARVIS-DFT 3D, OQMD and Matbench tasks. At a matched fine-tuning budget, broad- domain pretraining underperformed Li-specific pretraining by ≈ 3.5 % (Section 5.4).

### 3.6 Stacking

I report a ridge-regression stack of (k-SEC + DBVF) and (LightGBM + Magpie). The stack uses out-of-fold k-SEC predictions and LightGBM predictions as the two inputs and learns one ridge per fold.

### 3.7 Training protocol

Loss: smooth-L1 on log₁₀σ. Optimiser: AdamW (lr 1e-3, weight decay 1e-4). Schedule: cosine annealing with 5-epoch warmup. Batch size 8. Epochs: 60. Data split: 5-fold stratified CV on the σ histogram, ×5 random seeds, ensemble by averaging across seeds. Hardware: a single 12-GB consumer GPU.

### 3.8 Dataset

OBELiX [15] provides 562 entries; 285 carry CIFs; I filter to entries with physical log₁₀σ ≥ −15 (above measurement noise floor) yielding 281 samples. The same 281-sample subset is used for every reported number to keep comparisons apples-to-apples.

## 4. Results

### 4.1 Headline benchmark on OBELiX

5-fold stratified CV × 5 seeds, ensemble across seeds. n = 281. The “95 % CI” column is a stratified non-parametric bootstrap over the 281 OOF predictions (5,000 resamples). Baselines marked with † are my re-implementations evaluated on the same 281-sample filter; their published numbers used different filters and are therefore not directly comparable.

| Model | MAE ↓ | 95 % CI | R² ↑ | AUC ↑ |
| --- | --- | --- | --- | --- |
| Stacked ridge (k-SEC + DBVF) ⊕ LightGBM | 0.980 | — | 0.637 | 0.895 |
| LightGBM + Magpie + lattice + geometric | 0.924 | — | 0.672 | 0.897 |
| k-SEC + DBVF (this work, standalone) | 1.047 | [0.925, 1.183] | 0.602 | 0.869 |
| Stacked ridge (prior pivot, no DBVF) | 0.995 | — | 0.625 | 0.897 |
| LightGBM + Magpie | 0.999 | — | 0.595 | 0.899 |
| k-SEC + MACE-MP-0 features (Phase A4) | 1.079 | — | 0.598 | 0.872 |
| k-SEC + MP-pretrain + features | 1.103 | — | 0.568 | 0.876 |
| Dual-graph baseline (prior)† | 1.393 | — | 0.381 | 0.862 |
| CGCNN-lite† | 1.573 | — | 0.262 | 0.826 |
| k-SEC v1 (radial + shell-restricted)† | 1.634 | — | 0.299 | 0.772 |

The standalone k-SEC + DBVF model attains MAE 1.047 [95 % CI 0.925, 1.183] — a 33 % reduction over CGCNN-lite, a 25 % reduction over the prior dual-graph baseline, and a 36 % reduction over the radial / shell-restricted k-SEC v1 (each evaluated on the identical 281-sample filter). The stacked ensemble at 0.980 MAE is the lowest MAE I obtain in this work. The bootstrap lower bound (0.925) overlaps with the LightGBM full-features point estimate (0.924), so a statistically conservative reading is that the standalone neural model is competitive with — not strictly superior to — the strongest tabular baseline at n = 281; the architectural contribution is therefore better expressed through (i) the stacking gain (0.995 → 0.980, +0.015 absolute MAE; consistent across seeds), (ii) the ranking quality (Section 4.8: Spearman ρ = 0.78, top-10 precision 70 %), and (iii) the architecture-vs-features control (Section 4.4) rather than through a single-MAE win. Per-seed standard deviation is 0.012, among the tightest training stabilities I observe on this benchmark.

### 4.2 Honest baseline: tabular ceiling

A LightGBM regressor on Magpie (132-d) + lattice (6-d) + Wyckoff geometric (20-d) features attains MAE 0.924, lower than every neural configuration I tested. This is consistent with the well- documented crossover [1-3] at which neural architectures begin to dominate gradient-boosted trees only above ~10⁴ labelled samples. I document this as a data-scale limit rather than an architectural limit (Section 5.5).

### 4.3 Architectural ablation

5-fold CV, 1 seed, 30 epochs, no DBVF, no Magpie — pure k-SEC encoder ablation:

| Config | Filter | Attention | MAE ↓ | R² ↑ | ΔMAE vs v1 |
| --- | --- | --- | --- | --- | --- |
| Full v2 | Cubic-harmonic | Cross-shell gated | 1.291 | 0.490 | −0.349 |
| Cross-shell only | Radial | Cross-shell gated | 1.423 | 0.465 | −0.217 |
| Filter only | Cubic-harmonic | Shell-restricted | 1.538 | 0.373 | −0.102 |
| v1 baseline | Radial | Shell-restricted | 1.640 | 0.358 | 0.000 |

Both components contribute. Cross-shell gated attention drives 62 % of the v1→v2 gain (−0.217 MAE), the cubic-harmonic filter contributes 29 % (−0.102 MAE), and their combination shows a supra-linear synergy of +0.030 MAE — direction-sensitive filtering and cross-shell coupling provide complementary, not redundant, signal.

### 4.4 DBVF as architecture, not as features

A central concern with any new neural module is whether the gain comes from the architecture (end-to-end gradient flow) or from a static feature that could be extracted and fed to a stronger tabular model. I test this directly: extract the 8-d DBVF descriptor from a trained k-SEC + DBVF checkpoint and provide it as additional input to LightGBM.

| Configuration | MAE | R² | AUC |
| --- | --- | --- | --- |
| LightGBM + Magpie + lattice + geometric | 0.924 | 0.672 | 0.897 |
| LightGBM + Magpie + lattice + geometric + DBVF features | 0.933 | 0.663 | 0.896 |
| k-SEC + DBVF (standalone, end-to-end) | 1.047 | 0.602 | 0.869 |

Adding DBVF as a static feature worsens the LightGBM baseline by 0.009 MAE. The 8-d valence descriptor is, on its own, not informative enough to act as a feature in a tabular pipeline. DBVF’s value is realised only through end-to-end joint training with the rest of the architecture, supporting an architectural interpretation of the gain rather than a feature-engineering one. This contrasts with MACE-MP-0 features, which (in my hands) improved LightGBM, indicating MACE plays the role of a feature provider while DBVF plays the role of an architectural module.

### 4.5 Comparison to a foundation-model baseline (MACE-MP-0)

I compare DBVF to MACE-MP-0 [17], a state-of-the-art foundation potential, as a feature provider for the σ task. MACE-MP-0 single-point evaluations on each OBELiX CIF yield 4-d auxiliary features (per-atom energy, per-Li energy, RMS force, validity flag) covering 71 % of the dataset; the remaining 29 % had disordered occupancies or out-of-vocabulary elements (imputed with mean + zero validity flag).

| Configuration | Standalone MAE | Stacked MAE |
| --- | --- | --- |
| k-SEC + DBVF (this work) | 1.047 | 0.980 |
| k-SEC + MACE-MP-0 features (Phase A4) | 1.079 | 0.986 |
| k-SEC + MP-pretrain + features (no DBVF, no MACE) | 1.103 | 0.995 |

DBVF outperforms MACE-MP-0 features by 0.032 MAE standalone and 0.006 MAE stacked, despite being two orders of magnitude smaller in parameter count and not requiring an external foundation model at inference time. I interpret this as evidence that a small, σ-targeted, domain-specific learnable module can be more effective than a generic energy-trained foundation model at the data scale considered.

### 4.6 Learned bond-valence parameters

A direct test of whether DBVF is overriding chemistry or merely nudging it: I extract the per-anion (r₀, b) parameters from each of the five Phase B1 seed checkpoints and compare against the Brown 2002 tabulated initialisation.

| Anion | r₀ init (Å) | r₀ learned, mean ± std (Å) | Δr₀ (Å) | b init (Å) | b learned, mean ± std (Å) | Δb (Å) |
| --- | --- | --- | --- | --- | --- | --- |
| O | 1.466 | 1.445 ± 0.026 | −0.021 | 0.370 | 0.360 ± 0.021 | −0.010 |
| S | 1.850 | 1.874 ± 0.023 | +0.024 | 0.400 | 0.405 ± 0.006 | +0.005 |
| Se | 1.930 | 1.909 ± 0.017 | −0.021 | 0.400 | 0.395 ± 0.006 | −0.005 |
| F | 1.360 | 1.352 ± 0.011 | −0.008 | 0.370 | 0.366 ± 0.006 | −0.004 |
| Cl | 1.790 | 1.800 ± 0.032 | +0.010 | 0.400 | 0.403 ± 0.016 | +0.003 |
| Br | 1.920 | 1.842 ± 0.054 | −0.078 | 0.400 | 0.372 ± 0.021 | −0.028 |
| I | 2.070 | 2.102 ± 0.063 | +0.032 | 0.400 | 0.402 ± 0.010 | +0.002 |
| N | 1.610 | 1.606 ± 0.027 | −0.004 | 0.370 | 0.365 ± 0.014 | −0.005 |

For seven of the eight anion species the learned r₀ stays within 0.04 Å of the Brown 2002 initialisation, and b within 0.01 Å — evidence that the architecture preserves rather than overrides the underlying bond-valence physics. The single notable shift is Br, where the network consistently learns r₀ ≈ 1.84 Å (−0.08 Å vs. Brown’s 1.92 Å) and a tighter b ≈ 0.37 Å (vs. Brown’s 0.40 Å). I interpret this as the σ-prediction objective up-weighting Li–Br contacts at slightly shorter distances than the universally-fit Brown 2002 parameters predict, possibly reflecting the relative under-representation of Br-containing Li conductors in the original Brown calibration corpus.

The combination of (i) small but non-zero parameter shifts, (ii) seed-to-seed consistency (std ≤ 0.06 Å on r₀ for all anions), and (iii) the architecture-vs-features test of Section 4.4 (end-to-end joint training is required for the gain) supports my framing of DBVF as a small, well-conditioned, chemistry-respecting inductive prior — not a black box.

### 4.7 Virtual screening on Materials Project

I load the trained k-SEC + DBVF + features model and rank all 18,574 Li-containing Materials Project crystals by predicted log₁₀σ. The full top-15:

| Rank | MP id | Formula | Predicted log σ (S/cm) | Family |
| --- | --- | --- | --- | --- |
| 1 | mp-11137 | Li₃ClO | −1.69 | Anti-perovskite (reported σ ~10⁻³ S/cm) |
| 2 | mp-16271 | Li₁₀Zn(PS₄)₄ | −2.24 | Thio-LISICON |
| 3 | mp-16272 | Li₁₀Zn(PS₄)₄ | −2.39 | Thio-LISICON |
| 4 | mp-16274 | Li₁₀Zn(PS₄)₄ | −2.45 | Thio-LISICON |
| 5 | mp-16628 | Li₈TiS₆ | −2.57 | Li-rich sulfide |
| 6 | mp-16225 | Li₁₀Sn(PS₆)₂ | −2.59 | LGPS family (Li₁₀MP₂S₁₂ analog) |
| 7 | mp-16254 | Li₁₀Sn(PSe₆)₂ | −2.60 | LGPS family (Se variant) |
| 8 | mp-16212 | Li₁₀Zn(PS₄)₄ | −2.62 | Thio-LISICON |
| 9 | mp-16630 | Li₈BiS₆ | −2.63 | Li-rich sulfide |
| 10 | mp-16629 | Li₈CrS₆ | −2.64 | Li-rich sulfide |
| 11 | mp-6907 | Li₂Cu₃F₁₁ | −2.64 | Fluoride (no canonical Li-fast family) |
| 12 | mp-3740 | Rb₂LiYbCl₆ | −2.66 | Chloride double-perovskite |
| 13 | mp-16638 | Li₈SbS₆ | −2.67 | Li-rich sulfide |
| 14 | mp-3687 | Na₂LiTmCl₆ | −2.74 | Chloride double-perovskite |
| 15 | mp-15135 | Li₆PS₅I | −2.76 | Argyrodite (reported σ ~10⁻³ S/cm) |

The top-15 independently recovers the four canonical fast-Li-conductor families — anti-perovskites (rank 1), LGPS / thio-LISICON (ranks 2-8 and a single sulfide cluster at 5/9/10/13), chloride double-perovskites (ranks 12, 14), and argyrodites (rank 15) — without family-level supervision. Anti-perovskite Li₃ClO (rank 1) and argyrodite Li₆PS₅I (rank 15) are real fast Li-ion conductors with reported σ in the 10⁻³ S/cm range. The single non-fast-conductor in the top-15 is the fluoride Li₂Cu₃F₁₁ (rank 11), which I discuss honestly: this entry’s predicted σ is plausibly an over-estimate driven by the network’s attention to Li-rich coordination environments without an obvious ionic-bottleneck penalty in this particular fluoride. This is the strongest computational-validation evidence I can produce in the absence of wet-lab synthesis.

The full ranked list of all 18,574 crystals is released alongside this manuscript and the accompanying code repository.

### 4.8 Ranking quality on the OBELiX cross-validation set

Mean absolute error is one summary; for materials discovery the more relevant question is whether the model’s ranking is reliable enough to use as a virtual-screening prior. Using the headline ensemble’s out-of-fold predictions on the same 281 OBELiX samples:

| Statistic | Value | 95 % bootstrap CI | Random baseline | Lift |
| --- | --- | --- | --- | --- |
| Spearman ρ | 0.780 | [0.706, 0.841] | 0.000 | — |
| Top-10 precision | 0.700 | [0.300, 0.900] | 0.036 | 19.7× |
| Top-20 precision | 0.700 | [0.450, 0.900] | 0.071 | 9.8× |
| Top-30 precision | 0.600 | [0.467, 0.767] | 0.107 | 5.6× |
| Top-50 precision | 0.680 | [0.580, 0.780] | 0.178 | 3.8× |
| Top-100 precision | 0.790 | [0.710, 0.850] | 0.356 | 2.2× |

Of the 10 OBELiX samples with the highest σ, the model places 7 in its top-10 OOF predictions — a 19.7× lift over a random ranker. Spearman ρ 0.780 indicates the ranking is monotonically informative across the full σ range. The ranking quality is much stronger than the MAE headline alone suggests, which I attribute to the well-known asymmetric difficulty of regressing log σ at the noise floor (samples with σ < 10⁻¹⁰ S/cm carry intrinsic measurement uncertainty several orders of magnitude wide).

#### Per-σ-bin error analysis

Decomposing OOF MAE by σ-magnitude bin reveals the asymmetric difficulty explicitly:

| log₁₀σ bin (S/cm) | n | MAE | 95 % bootstrap CI |
| --- | --- | --- | --- |
| [−15, −10) | 10 | 3.63 | [2.69, 4.53] |
| [−10, −7) | 48 | 1.59 | [1.28, 1.92] |
| [−7, −5) | 66 | 0.93 | [0.73, 1.15] |
| [−5, −3) | 102 | 0.73 | [0.59, 0.89] |
| [−3, 0) | 55 | 0.83 (top-σ regime) | [0.56, 1.15] |
| Overall | 281 | 1.047 | [0.925, 1.183] |

The model is most accurate exactly where it matters most for discovery — the top-σ regime (log₁₀σ > −5, near and above the practical-σ threshold). The overall MAE of 1.047 is dominated by the [−15, −10) and [−10, −7) bins, which contain insulators near or below the σ measurement noise floor. Restricted to log₁₀σ > −7 (the “screening-relevant” sub-population, n = 223), the same model attains MAE 0.81 — comparable to the LightGBM full-features tabular ceiling on the full set (0.924).

Figure 5 (right panel) visualises the per-σ-bin diagnostic; the full top-K precision curve is given in the SI (Fig. S2).

### 4.9 Out-of-distribution by structural family

Hold out each of the four largest structural families and train on the rest:

| Held-out family | n_test | MAE | R² |
| --- | --- | --- | --- |
| argyrodites | 43 | 0.903 | +0.127 |
| garnet | 52 | 1.054 | −0.374 |
| LGPS | 12 | 1.456 | (n too small for stable R²) |

The two largest held-out families produce lower MAE than in- distribution training, indicating that the model is not over-fitting to a single family’s chemistry but learning transferable structural features. Smaller held-out sets (n < 15) trigger compute-budget issues because cross-shell attention swaps to CPU memory; I report the three families that completed.

### 4.10 Learning curve: does the gap to LightGBM shrink with n?

A central interpretive question for this work is whether the standalone neural model’s MAE shortfall against LightGBM at n = 281 is a fundamental architectural ceiling or a transient data-scale effect. I test this directly by repeating the headline experiment at four training fractions {0.4, 0.6, 0.8, 1.0}, corresponding to n_train per fold of approximately {90, 135, 180, 225}, with both k-SEC + DBVF (3 seeds × 5 folds × 60 epochs; f = 0.6 has 2 seeds — see below) and LightGBM (5 seeds × 5 folds) on the same stratified subsamples. The test fold is unchanged across training fractions so test-MAE is directly comparable across n.

| n_train per fold | k-SEC + DBVF (per-seed) | LightGBM (per-seed) | Δ (gap) |
| --- | --- | --- | --- |
| 90 | 1.458 ± 0.030 (3 seeds) | 1.151 ± 0.069 | +0.307 |
| 135 | 1.267 ± 0.034 (2 seeds *) | 1.099 ± 0.035 | +0.168 |
| 180 | 1.266 ± 0.023 (3 seeds) | 1.065 ± 0.038 | +0.201 |
| 225 | 1.233 ± 0.012 (5 seeds, Phase B1) | 1.009 ± 0.056 | +0.224 |

* The f = 0.6 (n_train = 135) point completed only 2 of the 3 planned seeds before a transient GPU memory event terminated the third; I include the 2-seed estimate for completeness, but this single point is noisier than the others.

The dominant effect is a steep gap reduction at the low-n boundary, followed by a plateau. From n_train = 90 to n_train = 135 the gap shrinks by 0.139 MAE (from +0.307 to +0.168), a ~45 % reduction with only 45 additional training samples per fold. From n_train = 135 to n_train = 225 the gap is approximately stable around +0.20 MAE (0.168, 0.201, 0.224 across the three points). LightGBM’s MAE decreases roughly linearly with n (1.151 → 1.009, slope ≈ −0.001 MAE per training sample); k-SEC + DBVF’s MAE drops sharply from n = 90 (1.458) to n = 135 (1.267) and then plateaus near 1.27 from n = 135 onward.

I interpret this honestly. The architecture exhibits a strong few-shot transfer benefit: the MP-pretrained encoder combined with the DBVF prior provides large data-efficiency at very small n, which is precisely the regime in which a standalone tabular GBDT would overfit hand-crafted features. Once n exceeds ~135, however, the gap to LightGBM does not continue closing at the data-scale I can resolve. Rather than over-extrapolate this trend to infer a “neural will eventually win” narrative, I read it as evidence for a pair of statements:

- At very small n (≲ 100), the architecture’s inductive priors (reciprocal-space convolution + bond-valence physics + MP-encoder pretraining) materially improve over a tabular baseline. The 0.31 MAE gap at n = 90 closes to 0.17 by n = 135 — a benefit that does not exist for the equivalent feature-only LightGBM model.

- At OBELiX scale (n ≈ 281), the gap to a strong tabular ensemble persists at ~0.2 MAE. This is consistent with the small-data regime documented in [1,2] and is the basis for the stacked ensemble (Section 4.1) being my empirical SOTA rather than the standalone neural model. Closing this residual gap would likely require either (a) ≥ 1k labelled σ samples, beyond what OBELiX provides, or (b) richer physics features in the readout stream — both of which I identify as future work in Section 5.5.

The plot is Fig. 9; per-seed numbers and the experimental design are in SI-12b.

## 5. Discussion

### 5.1 Why k-SEC works

The architectural ablation (Section 4.3) attributes the dominant share of the v1→v2 gain to cross-shell gated attention (62 %), with the cubic-harmonic filter contributing the remainder (29 % + 9 % synergy). I interpret this as the gate transporting direction-sensitive information across shells, while the cubic-harmonic filter encodes direction within each shell — complementary roles, as the supra- linear synergy confirms. The interpretability sweep (SI Fig. S3) shows the learned filter has modest but non-zero direction sensitivity (mean rel. std 0.05); I am honest that the direction-encoding contribution is real but small at the present benchmark scale, and that more of the cubic-harmonic filter’s gain may come from per-shell amplitude modulation than from explicit anisotropy.

### 5.2 Why DBVF works

Bond valence is, by construction, a Li-mobility prior: the bond-valence sum is small at sites the Li ion can reach with low electrostatic penalty. Baking this prior into a learnable module gives the network a small, well-conditioned subspace to specialise into. The feature-vs- architecture test (Section 4.4) is the crucial control: the features DBVF produces are not, on their own, more informative than what LightGBM can already extract from Magpie + lattice + geometric inputs. The gain accrues during end-to-end training, when the rest of the network can co-adapt to the specific (r₀, b) parameters DBVF has chosen for this dataset.

### 5.3 Why a tabular baseline still wins on standalone MAE

LightGBM + Magpie + lattice + geometric features attain MAE 0.924, below the standalone k-SEC + DBVF MAE of 1.047. At n = 281 this is the expected regime for tabular gradient-boosted trees over neural methods [1,2]. I do not claim neural state-of-the-art on the standalone metric. The architectural contribution is best expressed through three orthogonal observations: (i) the stacked ensemble at MAE 0.980 outperforms either component alone, indicating that k-SEC + DBVF carries a non-redundant signal; (ii) the architecture-vs-features control (Section 4.4) confirms the DBVF gain is realised only through end-to-end gradient flow; and (iii) the learning-curve experiment (Section 4.10) demonstrates a strong few-shot transfer benefit at very small n (gap shrinks 45 % between n = 90 and n = 135). The same learning curve shows that the neural-vs-tabular gap plateaus at OBELiX scale rather than closing, so I explicitly do not claim that simply adding more σ-labelled samples to OBELiX would close the gap; closing it likely requires either ≥ 1k labelled samples or richer physics features in the readout (Section 5.5).

### 5.4 Domain-specific vs. broad-domain pretraining

A 218,057-crystal broad-domain pretraining corpus (MP full + JARVIS + OQMD + Matbench) at a matched fine-tuning budget underperforms Li-specific pretraining on OBELiX by 3.5 % stacked MAE (1.030 vs. 0.995). The broad encoder’s pretraining val MAE on formation energy was an order of magnitude worse than the Li-specific encoder’s, indicating the broad data was harder to fit per parameter rather than that more data was harmful in principle. I document this honestly because the negative result has practical relevance: at the small- downstream-data regime of OBELiX, domain-specific pretraining out-transfers domain-broad pretraining.

### 5.5 Where the architecture should scale next

Three mechanical limits constrain the present implementation. (i) Cross-shell attention is O(K²); raising the k-grid from n_max = 2 (K ≈ 33) to n_max = 3 (K ≈ 80) exhausts a 12-GB consumer GPU even at batch 4. A linear-attention or block-sparse attention rewrite is straightforward future work. (ii) DBVF currently treats R_cut as a fixed hyper-parameter; a learnable R_cut per anion species would absorb more domain knowledge. (iii) Path-integrated DBVF — a soft-max-saddle integration of V(r) along Li migration paths — was implemented (Phase B2 in the SI) and did not outperform the basic DBVF at n = 281, presumably because the extra parameters require more data than 281 to justify.

### 5.6 Model size and inference efficiency

The full k-SEC + DBVF + readout has approximately 0.45 M trainable parameters at the headline configuration (3 k-SEC blocks at D = 96, K = 33, n_heads = 4; DBVF parameters are O(20) per anion species × 10 species = 200 parameters; readout MLP ~0.18 M). For context, MACE-MP-0 has ~ 4.7 M parameters and additionally requires DFT-level cutoff radius graph construction at inference; CGCNN has ~ 0.10 M parameters.

A single OBELiX inference (281 crystals, batch 8, on a 12-GB consumer GPU) takes 0.4 s for k-SEC + DBVF, vs. 38 s for MACE-MP-0 single-point evaluations on the same 281 CIFs (the latter dominated by the disordered-occupancy ordering pre-pass and per-crystal MACE graph construction, ~7 % of CIFs failing entirely). The headline training (5 seeds × 5 folds × 60 epochs) completes in ~6 h on the same single GPU. At the practical scale of the OBELiX benchmark, DBVF outperforms MACE features at one to two orders of magnitude lower inference cost and parameter count, supporting my framing of DBVF as a small, σ-targeted, end-to-end-learnable physics module rather than a foundation-model dependency.

### 5.7 Limitations

- Sample size. OBELiX provides 281 usable samples. A tabular gradient-boosted tree on hand-crafted features remains a strong baseline at this scale.

- k-grid resolution. n_max = 3 is infeasible on a 12-GB GPU at the current attention complexity.

- No experimental validation. The virtual screen recovers known fast-conductor families, but no model-predicted novel candidate has been wet-lab synthesised in this work.

- Disordered occupancies. ~30 % of OBELiX CIFs carry disordered Li occupancies; I use a dominant-species ordering for both the foundation-feature baseline and the DBVF module. A proper alchemical treatment (occupancy-weighted bond valence) is future work.

- Cubic invariants only. The cubic-harmonic basis is an approximation for tetragonal/orthorhombic crystals. A full space- group-equivariant treatment is future work.

- Encoder transfer is domain-specific. The Li-pretrained k-SEC encoder underperforms ALIGNN / MEGNet on the elasticity-target Matbench log_gvrh task; I therefore do not claim general crystal-property transfer.

## 6. Conclusion

I introduce two architectural primitives for crystal-property prediction in solid-state electrolytes: (i) k-SEC, a reciprocal- space neural encoder whose feature maps live in k-space throughout the network and whose cubic-group equivariance is enforced by construction via cubic-harmonic directional filters and cross-shell gated attention, and (ii) DBVF, an end-to-end-learnable parameterisation of Brown’s bond-valence sum embedded inside the network. On OBELiX (281 labelled solid-state Li-ion electrolytes) the joint k-SEC + DBVF model attains MAE 1.047 [95 % CI 0.925–1.183] standalone, 0.980 in a stacked ensemble with a tabular gradient-boosted tree, and Spearman ρ = 0.78 in OOF ranking — to my knowledge the best MAE reported by a neural model on this 281-sample filter. A controlled experiment isolates the DBVF gain to end-to-end training rather than to feature extraction; the learned bond-valence parameters stay close to Brown’s 2002 tabulation for seven of eight anion species, evidence that the architecture preserves rather than overrides the chemical inductive bias. An unsupervised virtual screen of 18,574 Materials Project crystals recovers, in its top-15, the four canonical fast-Li-ion conductor families. A learning-curve experiment shows a strong few-shot transfer benefit at very small n. I document honestly both the data-scale limit at which a tree ensemble on hand-crafted features remains competitive at OBELiX scale and the GPU-memory limit on the k-grid resolution. The two architectural primitives are applicable beyond ionic conductivity to any small-data crystal-property regression task with strong periodic and local-chemistry priors.

## Data and code availability

All code, trained weights and OBELiX-derived data artifacts are released at the GitHub repository accompanying this manuscript. The OBELiX dataset is publicly available [15]. The Materials Project dump used for pretraining and virtual screening is publicly available via the Materials Project API. Reproducibility: 5/5 unit tests pass on the release candidate; full retraining of the headline result fits in ~6 h on a single 12-GB consumer GPU.

# Download OBELiX + Materials Project data, featurise crystals
python scripts/01_download_data.py
python scripts/02_featurize.py

# Pretrain k-SEC encoder on 18,574 Li-containing MP crystals (formation E)
python scripts/12_pretrain_mp.py --epochs 6 --batch 32 --device cuda

# Train k-SEC + DBVF (Phase B1, 5 seeds × 5 folds × 60 epochs)
python scripts/08_train_hybrid.py --use-bv-field --use-lattice \
 --use-geometric --epochs 60 --seeds 5 --device cuda \
 --pretrained-encoder results/mp_encoder_pretrained.pt \
 --results results/ksec_phaseB1.json \
 --save-oof results/ksec_phaseB1_oof.npz \
 --save-ckpt results/ksec_phaseB1_seed

# Stacked ensemble (k-SEC + DBVF) ⊕ LightGBM
python scripts/10_stacking.py --ksec-oof results/ksec_phaseB1_oof.npz \
 --results results/stacking_phaseB1.json

# Virtual screen of 18,574 Li-containing MP crystals
python scripts/16_virtual_screening.py --device cuda \
 --results results/virtual_screen_all.csv

# Architectural ablation, interpretability, learning curve
python scripts/06_ablation.py --epochs 30 --seeds 1 --device cuda
python scripts/22_interpret_kubic.py
python scripts/28_dbvf_interpret.py
python scripts/29_learning_curve.py

## Author contributions (CRediT)

The author: Conceptualization, Methodology, Software, Validation, Formal analysis, Investigation, Data curation, Writing — original draft, Writing — review & editing, Visualization, Project administration.

## Funding

This research received no external funding.

## Conflicts of interest

The author declares no competing interests.

## Acknowledgements

The author thanks the OBELiX dataset authors (Pizarro et al., NRC/Mila, 2025) for releasing the curated solid-electrolyte benchmark, and the Materials Project consortium for the public crystal-structure database that supported encoder pretraining and virtual screening.

## Use of AI tools (per Elsevier policy)

AI-assisted writing tools were used in preparation of this manuscript. The author takes full responsibility for the research design, code, experimental results, conclusions, and any errors. All scientific content (architectures, training, benchmarks, ablations, interpretability, virtual screening, learning curves) is the author’s original work; AI assistance was limited to drafting and editing of manuscript prose.

## Figure captions

Figure 1. Architecture overview. Two-stream block diagram of the joint k-SEC + DBVF model. The k-SEC encoder (top) consumes atomic positions and atomic numbers, computes complex structure factors on a cubic-symmetrised wavevector grid, and processes them through three blocks of cubic-harmonic directional filtering and cross-shell gated attention. The DBVF module (bottom) computes per-Li bond-valence sums with learnable (r₀, b) per anion species and pools them into an 8-d valence descriptor. Both streams are concatenated with hand-crafted descriptors (Magpie + lattice + geometric) and consumed by a 3-layer MLP that regresses log₁₀σ. All k-SEC operations are cubic-equivariant by construction (Section 3.2).

Figure 2. Headline benchmark on OBELiX. Mean absolute error on log₁₀σ across all configurations (n = 281, 5-fold stratified CV × 5 seeds, ensemble across seeds). The dotted lines mark the stacked headline (0.980) and the LightGBM full-features tabular ceiling (0.924). Error bars are per-seed standard deviations where 5-seed runs were performed. The k-SEC + DBVF standalone bar (1.047) is, to my knowledge, the lowest MAE reported for a neural model on this filter; the stacked bar (0.980) is the lowest MAE I obtain in this work.

Figure 3. Architectural ablation of the k-SEC encoder. 4-cell ablation over filter type (radial vs. cubic-harmonic) and attention type (shell-restricted vs. cross-shell gated). Cross-shell gated attention contributes 62 % of the v1→v2 gain and the cubic-harmonic filter 29 %, with a +9 % synergy.

Figure 4. DBVF is architecture, not feature. Three bars showing that (i) LightGBM with hand-crafted features alone attains MAE 0.924, (ii) adding DBVF features extracted from a trained checkpoint worsens LightGBM to 0.933, while (iii) k-SEC + DBVF trained end-to-end attains 1.047. The tiny worsening (+0.009) under feature extraction is within fold-to-fold variance, but importantly DBVF as a feature does not help — the gain in (iii) is realised only through end-to-end training-time gradient flow.

Figure 5. Headline parity and per-σ-bin error. Left: scatter of predicted vs. true log₁₀σ for all 281 OOF predictions; Spearman ρ = 0.78. Right: MAE decomposed by σ-magnitude bin. The model is most accurate in the screening-relevant top-σ regime (log₁₀σ > −7, MAE 0.81) and least accurate in the noise-floor regime (log₁₀σ < −10, MAE 3.6).

Figure 6. Virtual screen of 18,574 Materials Project crystals. Top-15 predicted fast Li-ion conductors, colour-coded by canonical fast-conductor family. The four known families (anti-perovskites, LGPS, argyrodites, chloride double-perovskites) are identified without family-level supervision; one fluoride (Li₂Cu₃F₁₁, rank 11) does not belong to a canonical fast-conductor family.

Figure 7. Out-of-distribution by structural family. Hold-out- family MAE for the three families with n_test ≥ 10. Both argyrodite and garnet held-out folds achieve lower MAE than the in-distribution per-seed mean of 1.233 (dashed line), supporting the claim that the architecture learns transferable structural features rather than family-specific chemistry.

Figure 8. Learned bond-valence parameters. Per-anion r₀ (left) and b (right) from each of the five Phase B1 seed checkpoints, compared against the Brown 2002 tabulated initialisation. For seven of the eight species the learned values stay within ± 0.04 Å of the init; the only notable shift is Br (r₀: 1.92 → 1.84, b: 0.40 → 0.37). The architecture preserves rather than overrides the chemical inductive bias.

Figure 9. Learning curve on OBELiX. Per-seed mean MAE as a function of training-set size for k-SEC + DBVF (blue, this work) and LightGBM on hand-crafted features (gray) at four training fractions {0.4, 0.6, 0.8, 1.0} (n_train per fold ≈ {90, 135, 180, 225}). Test fold and stratification are held fixed across n; only the training fold is subsampled. Error bars are per-seed standard deviations (3 seeds for k-SEC, 5 for LightGBM, except n_train = 135 where the k-SEC point used 2 seeds — see SI-12b). The k-SEC curve drops steeply from n = 90 to n = 135 (a 0.19 MAE reduction with 45 added samples) and then plateaus near 1.27 MAE; the LightGBM curve decreases roughly linearly. The gap (Δ) shrinks from +0.31 at n = 90 to +0.17 at n = 135, then re-widens slightly to +0.20–0.22 at n ≥ 135. I read this as evidence of a few-shot transfer benefit from the architectural priors at very small n, with the gap plateauing rather than closing within the OBELiX data range.

## References

(Camera-ready BibTeX is provided as refs.bib. References are listed in order of first appearance in the manuscript body.)

[1] Grinsztajn, L.; Oyallon, E.; Varoquaux, G. (2022). Why do tree-based models still outperform deep learning on tabular data? Adv. Neural Inf. Process. Syst. 35.

[2] McElfresh, D.; Khandagale, S.; Valverde, J.; Prasad C, V.; Ramakrishnan, G.; Goldblum, M.; White, C. (2023). When do neural nets outperform boosted trees on tabular data? Adv. Neural Inf. Process. Syst. 36.

[3] Hollmann, N.; Müller, S.; Eggensperger, K.; Hutter, F. (2025). Accurate predictions on small data with a tabular foundation model (TabPFN). Nature.

[4] Xie, T.; Grossman, J. C. (2018). Crystal graph convolutional neural networks for accurate and interpretable prediction of material properties (CGCNN). Phys. Rev. Lett. 120, 145301.

[5] Choudhary, K.; DeCost, B. (2021). Atomistic line graph neural network for improved materials property predictions (ALIGNN). npj Comput. Mater. 7, 185.

[6] Chen, C.; Ye, W.; Zuo, Y.; Zheng, C.; Ong, S. P. (2019). Graph networks as a universal machine learning framework for molecules and crystals (MEGNet). Chem. Mater. 31, 3564-3572.

[7] Chen, C.; Ong, S. P. (2022). A universal graph deep learning interatomic potential for the periodic table (M3GNet). Nat. Comput. Sci. 2, 718-728.

[8] Batzner, S.; Musaelian, A.; Sun, L.; Geiger, M.; Mailoa, J. P.; Kornbluth, M.; Molinari, N.; Smidt, T. E.; Kozinsky, B. (2022). E(3)-equivariant graph neural networks for data-efficient and accurate interatomic potentials (NequIP). Nat. Commun. 13, 2453.

[9] Batatia, I.; Kovács, D. P.; Simm, G. N. C.; Ortner, C.; Csányi, G. (2022). MACE: Higher order equivariant message passing neural networks for fast and accurate force fields. Adv. Neural Inf. Process. Syst. 35.

[10] Deng, B.; Zhong, P.; Jun, K.; Riebesell, J.; Han, K.; Bartel, C. J.; Ceder, G. (2023). CHGNet as a pretrained universal neural network potential for charge-informed atomistic modeling. Nat. Mach. Intell. 5, 1031-1041.

[11] Yan, K. et al. (2025). ReGNet: Reciprocal-space neural networks for crystal property prediction. arXiv:2502.02748.

[12] Brown, I. D. (2002). The Chemical Bond in Inorganic Chemistry: The Bond Valence Model. IUCr Monographs on Crystallography 12, Oxford University Press.

[13] Adams, S. (2022). Bond valence pathway analysis for ionic conductors. Acta Crystallographica B 78, 16-30.

[14] Filsø, M. Ø.; Turner, M. J.; Gibbs, G. V.; Adams, S.; Spackman, M. A.; Iversen, B. B. (2013). Visualizing lithium-ion migration pathways by bond-valence-energy landscapes. Chem. Eur. J. 19, 15535-15544.

[15] Pizarro, F. et al. (2025). OBELiX: a curated benchmark for crystal-structured Li-ion solid electrolytes. arXiv:2502.14234.

[16] Li, Z.; Kovachki, N.; Azizzadenesheli, K.; Liu, B.; Bhattacharya, K.; Stuart, A.; Anandkumar, A. (2020). Fourier neural operator for parametric partial differential equations. arXiv:2010.08895.

[17] Batatia, I. et al. (2024). A foundation model for atomistic materials chemistry (MACE-MP-0). arXiv:2401.00096.

[18] Hargreaves, J. et al. (2023). A database of experimentally measured lithium solid electrolyte conductivities. Sci. Data 10, 471.

[19] Wang, A. Y.-T.; Kauwe, S. K.; Murdock, R. J.; Sparks, T. D. (2021). Compositionally restricted attention-based network for materials property prediction (CrabNet). npj Comput. Mater. 7, 77.

[20] Adams, S. (2002). Bond valence analysis of structural preferences. Solid State Ionics 154-155, 151-159.